Electrical/Bio Engineering Case Study – Fear Analysis using GSR Data

Demarce Williams (dsw9740)

ENGR-UH 1000, Fall 2023

STEP 1: Problem Identification and. Statement

The objective of this assignment is to develop a software to find the Fear Index using a set of GSR Data values, and a sequence of filters. The program will calculate the Fear Index based on 10 features extracted from the GSR Data set and return the Fear Index value and GSR-time graphs representing the data.

STEP 2: Gathering of Information and Input and Output Description

The galvanic skin response (GSR) is a measure of skin conductance influenced by the autonomic nervous system's sympathetic branch, regulating sweating. Sympathetic arousal increases sweat gland activity, elevating skin conductance, and vice versa. GSR sensors, like the Grove GSR sensor, measure sweat gland activity, correlating with emotional arousal. The sensor utilizes two electrodes on fingers, converting analogue voltage to digital via an Arduino microcontroller, then transmitting data to a computer as shown in the figure below.

GSR data, measured in micro-Siemens, reflects skin conductance. Beyond emotional arousal monitoring, GSR is used in various fields, including psychology and polygraphs, for lie detection. This assignment involves processing GSR data in MATLAB to detect fear. The data acquisition system records approximately 130 seconds of data at 48 Hz, offering insights into emotional responses.

The fear index value is  calculated using the following features:

- Mean of GSR Signal (F1): The average value of the GSR signal over the recording period which indicates the central tendency of the skin conductance responses.

- Variance of GSR Signal (F2): Description: The measure of how much the GSR values deviate from the mean which reflects the overall variability or spread of the skin conductance responses.

- GSR Peak Rise Time Sum (F3): The sum of the rise times for all detected GSR peaks which represents the cumulative time it takes for the skin conductance to rise to its peak level from the preceding trough level.

- GSR Peak Amplitude Sum (F4): The sum of the amplitudes of all detected GSR peaks which indicates the cumulative magnitude of the skin conductance responses.

- GSR Peak Energy Sum (F5): The sum of the energies of all detected GSR peaks, calculated as 1/2 * amplitude * rise time which provides a measure that combines both the amplitude and rise time of the skin conductance responses.

- Amplitude of the Highest Skin Conductance Response (F6): The magnitude of the highest GSR peak in the recording which highlights the intensity of the most prominent skin conductance response.

- Rise Time of the Highest Skin Conductance Response (F7): The time it takes for the GSR signal to rise to its peak for the highest response.

- Number of GSR Peaks (F8): The count of distinct peaks in the GSR signal which indicates the frequency of skin conductance responses during the recording.

- Mean Power of GSR Signal (F9): The average power of the GSR signal, computed as the mean of the squared GSR values.

- GSR Bandwidth (F10): The GSR Bandwidth is a measure of the frequency content of the GSR signal. It is calculated using the formula:

GSR Bandwidth represents the distribution of frequency components in the GSR signal. A higher GSR Bandwidth may suggest a broader range of changes in skin conductance, reflecting varied physiological responses.

The fear index, which indicates the presence of fear, is then calculated using the following equation:

The inputs/outputs may be represented as follows:

The user upon starting the program will be presented with a menu that gives them six options and will be asked to select one of the six options.

Inputs

- GSR Data file, median order, N-point moving average order, passband frequency

Output

- Fear Index, GSR-time graphs

Alternative Block Diagram

GSR Data csv file

Median order

Fear Index

N-point moving average order

GSR-Time Graphs

Passband frequency

STEP 3: Test Cases and Algorithm Design

- Test Cases

The following data set is tested with a 3rd order median filter, 10-point moving average filter, and 20 Hz passband frequency for the lowpass filter.

Test case 1: GSR_FEAR.csv

  - Expectation about Fear Index: Anticipate a heightened fear index result for this dataset.

  - Graphical Representation of Smoothing: The graphs are expected to exhibit consistent smoothness as a result of the applied filter.

  - Graphical Representation of Fluctuations and Separations: The graph should display increased fluctuations and noticeable trough-peak separations.

Test case 2: GSR_Baseline.csv

  - Expectation about Fear Index: Anticipate a low fear index result for this dataset.

  - Graphical Representation of Smoothing: The graphs are expected to exhibit consistent smoothness as a result of the applied filter.

  - Graphical Representation of Fluctuations and Separations: The graph should display reduced fluctuations and negligible trough-peak separations.

- Pseudocode Algorithm

START

Assign GSR Data file with extension to file

Assign 3 to med_order

Assign 10 to n_order

Assign20 to fp

GSRData = CALL loadGSRData with argument: file

Assign return value from CALL  nthOrderMedianFilter with arguments: med_order, GSRData to filteredGSRData

Assign return value from CALL lowPassFilter with arguments: fp, filteredGSRData to filteredGSRData

Assign return value from CALL nPointMovingAverageFilter with arguments: n_order, filteredGSRData to filteredGSRData

Assign return value from CALL normaliseData with arguments: filteredGSRData to normalisedGSRData

Assign return value from CALL gsrFeatures with argument normalisedGSRData to F

Assign return value from CALL  calculateFearIndex with argument: F to FearIndex

Print newline, “The fear index is:”, FearIndex, newline

END

Function Definitions

Note the following format:

Module Name: Function name

Module Description:  Description of the purpose of function

ARGUMENTS: List of function arguments

PROCESS: Function definition goes here.

OUTPUT: State the return value of function, if any.

End Module: Function name, to signify end of function definition.

Each function used will begin on a new page.

Module Name: loadGSRData

Module Description: The following function loads GSR Data values from a file, passed as an argument and returns a matrix containing the data.

ARGUMENTS: data_file

PROCESS:

Open file, data_file, for reading

if file is equal to -1, then

Print “Error! Could not open file!”’ newline

end if

Skip header

Read time and gsr data from file in formats mm:ss:SSS and double, respectively,  delimited by a comma and store in data

Assign return value from CALL seconds(data{1}-data{1}(1)) to time

Close file

GSRData = {time, data{2}}

OUTPUT: GSRData

End Module: loadGSRData

Module Name: nthOrderMedianFilter

Module Description: The following function creates a nth order median filter for data set. It accepts two arguments: N-the median order and GSRData-the GSR data 3D cell array and returns a filtered 3D cell array.

ARGUMENTS: N, GSRData

PROCESS:

Assign GSRData  to filteredGSRData

for i = 1 to length(GSRData(2)), repeat

Assign return value from CALL  zeros(1, N) to subset

Assign 1 to k

for j = -(floor((N-1)/2)) to ceil((N-1)/2), repeat

Assign i + j to index

Assign return value from CALL max(1, min(index, length(GSRData(2))))

Assign GSRData{2}(index) to subset(k)

Increment k by 1

end for

Assign return value from CALL median(subset) to filteredGSRData{2}(i)

end for

Create new figure

Create 2 x 1 subplot

At subplot position 1

Plot GSRData{1} vs GSRData{2}

Set title "GRAPH OF GSR DATA BEFORE MEDIAN FILTRATION"

Set x label "Time (s)"

Set y label "GSR Values (uS)"

At subplot position 1

Plot  filteredGSRData{1} vs filteredGSRData{2} in red

Set title "GRAPH OF GSR DATA AFTER MEDIAN FILTRATION"

Set  x label "Time (s)"

Set  y label "GSR Values (uS)"

Saves current figure as "Med_Filt.jpg"

OUTPUT: filteredGSRData

End Module: nthOrderMedianFilter

Module Name: nPointMovingAverageFilter

Module Description: The following function creates a n-point moving average  filter for data set. It accepts two arguments: N-the moving average order and GSRData-the GSR data 3D cell array and returns a filtered 3D cell array.

ARGUMENTS: N, GSRData

PROCESS:

Assign GSRData  to filteredGSRData

for i = 1 to length(GSRData(2)), repeat

Assign return value from CALL  zeros(1, N) to subset

Assign 1 to k

for j = -(floor((N-1)/2)) to ceil((N-1)/2), repeat

Assign i + j to index

Assign return value from CALL max(1, min(index, length(GSRData(2))))

Assign GSRData{2}(index) to subset(k)

Increment k by 1

end for

Assign return value from CALL mean(subset) to filteredGSRData{2}(i)

end for

Create new figure

Create 2 x 1 subplot

At subplot position 1

Plot GSRData{1} vs GSRData{2}

Set title "GRAPH OF GSR DATA BEFORE N-POINT MOVING AVERAGE FILTRATION"

Set x label "Time (s)"

Set y label "GSR Values (uS)"

At subplot position 1

Plot  filteredGSRData{1} vs filteredGSRData{2} in red

Set title "GRAPH OF GSR DATA AFTER N-POINT MOVING AVERAGE FILTRATION"

Set  x label "Time (s)"

Set  y label "GSR Values (uS)"

Saves current figure as "Moving_Average.jpg"

OUTPUT: filteredGSRData

End Module: nPointMovingAverageFilter

Module Name: lowPassFilter

Module Description: The following function creates a lowpass filter for data set. It accepts two arguments: fp-the passband frequency and GSRData-the GSR data 3D cell array and returns a filtered 3D cell array.

ARGUMENTS: fp, GSRData

PROCESS:

Assign GSRData  to filteredGSRData

Assign GSRData to filteredGSRData

Assign return value from CALL diff(GSRData(1)) to timeDifference

Assign 1/fp to meanWindowSize

Assign return value from CALL round(meanWindowSize / median(timeDifference)) to meanWindowPoints

Assign return value from smoothdata(GSRData{2}, 'movmean', meanWindowPoints) to filteredGSRData{2}

Create new figure

Create 2 x 1 subplot

At subplot position 1

Plot GSRData{1} vs GSRData{2}

Set title "GRAPH OF GSR DATA BEFORE LOW PASS FILTRATION"

Set x label "Time (s)"

Set y label "GSR Values (uS)"

At subplot position 1

Plot  filteredGSRData{1} vs filteredGSRData{2} in red

Set title "GRAPH OF GSR DATA AFTER LOW PASS FILTRATION"

Set  x label "Time (s)"

Set  y label "GSR Values (uS)"

Saves current figure as "Med_Filt.jpg"

OUTPUT: filteredGSRData

End Module: lowPassFilter

Module Name: normaliseData

Module Description: The following function creates a lowpass filter for data set. It accepts two arguments: fp-the passband frequency and GSRData-the GSR data 3D cell array and returns a filtered 3D cell array.

ARGUMENTS: GSRData

PROCESS:

Assign GSRData  to normalisedData

for i = 1 to length(GSRData(2)), repeat

Assign  ((GSRData{2}(i) - min(GSRData{2})) / (max(GSRData{2})-min(GSRData{2})))* 100 to normalisedData{2}(i)

end for

Create new figure

Create 2 x 1 subplot

At subplot position 1

Plot GSRData{1} vs GSRData{2}

Set title "GRAPH OF GSR DATA BEFORE NORMALIZATION"

Set x label "Time (s)"

Set y label "GSR Values (uS)"

At subplot position 1

Plot  normalisedData{1} vs normalisedData{2} in red

Set title "GRAPH OF GSR DATA AFTER NORMALIZATION "

Set  x label "Time (s)"

Set  y label "GSR Values (uS)"

Saves current figure as "Med_Filt.jpg"

OUTPUT: normalisedData

End Module: normaliseData

Module Name: gsrFeatures

Module Description: The following function finds 10 features using GSR Data values from a file, passed as an argument and returns a vector containing the data..

ARGUMENTS:  GSRData

PROCESS:

Assign return value from CALL mean(GSRData{2}) to F(1)

Assign return value from CALL var(GSRData{2}) to F(2)

Assign return value from CALL islocalmax(GSRData{2}) to max_pk

Assign return value from CALL  islocalmin(GSRData{2}) to min_pk

Assign GSRData{2}(max_pk) to maxima

Assign GSRData{2}(min_pk)to minima

Assign GSRData{1}(max_pk) to max_t

Assign GSRData{1}(min_pk) to min_t

Assign 16.305 to tr_pk_sep_prctl

Assign 0 to F(3)

Assign 0 to F(5)

Assign return value from length(maxima) to SIZE

Assign return value from zeros(1, SIZE) to risetime

Assign return value from zeros(1, SIZE )to amplitude

for i = 1 to SIZE, repeat

Assign return value from find(min_t < max_t(i), 1, 'last') to idx

if idx is not empty

if maxima(i) - minima(idx) is greater than tr_pk_sep_prctl),then

Assign maxima(i) to amplitude(i)

Assign max_t(i) - min_t(idx) to risetime(i)

Assign F(3) + risetime(i) to F(3)

Assign F(5) + (0.5 * amplitude(i) * risetime(i)) to F(5)

end if

end if

end for

Assign return value from sum(amplitude) to F(4)

Assign return values from  max(amplitude) to F(6) and pk_idx

Assign risetime(pk_idx) to F(7)

Assign return value from CALL nnz(amplitude) to F(8)

Assign return value from CALL mean(GSRData(2).^2) to F(9)

Assign return value from CALL sum((diff(GSRData{2})).^2) to dSquares

Assign return value from CALL sum(GSRData{2}.^2) to squares

Assign return value from CALL sqrt(dSquares / squares) / (2 * pi) to F(10)

OUTPUT: F

End Module: gsrFeatures

Module Name: calculateFearIndex

Module Description: The following function finds the fear index using 10 features stored in a vector, passed as an argument, and returns a scalar of the fear index.

ARGUMENTS: F

PROCESS:

FearIndex = F(1) + (2*F(2)) + F(3) + (0.5*F(4)) + F(5) + (2*F(6)) + F(7) + (5*F(8)) + (0.001*F(9)) + (0.5*F(10))

OUTPUT: FearIndex

End Module: calculateFearIndex

STEP 4: Implementation

%The following program analyzes the relationship between the GSR signal and

%the physiological response to fear and returns a corresponding fear index.

file = 'GSR_Baseline.csv';

med_order = 3; %Median filter order

n_order = 10; %N-point moiving average order

fp = 20; %Passband frequency in Hz

%Loads GSR Data

GSRData = loadGSRData(file);

%Filters GSR Data

filteredGSRData = nPointMovingAverageFilter(n_order, lowPassFilter(fp, nthOrderMedianFilter(med_order, GSRData)));

%Normalises GSR Data to fit 100 range

normalisedGSRData = normaliseData(filteredGSRData);

%Extract Features from GSR Data and store in vector F

F = gsrFeatures(normalisedGSRData);

%Calculates Fear Index based on extracted features

FearIndex = calculateFearIndex(F);

%Displays the Fear Index

fprintf('\nThe fear index is: %.2f \n\n', FearIndex);

%The following function creates a nth order median filter for data set. It

%accepts two arguments: N-the median order and GSRData-the GSR data 3D cell

%array and returns a filtered 3D cell array.

function filteredGSRData = nthOrderMedianFilter(N, GSRData)

%Copy GSRData to filtered data array

filteredGSRData = GSRData;

%Loops through each GSR data value and replaces it with the median of

%teh values within the order window

for i = 1:length(GSRData{2})

% Determine the subset based on the specified order

subset = zeros(1,N); % Reserves memory for window GSR values

k = 1; %Index variable for subset array

for j = -floor((N-1)/2):ceil((N-1)/2)

index = i + j;

index = max(1, min(index, length(GSRData{2}))); % Ensure index is within bounds

subset(k) = GSRData{2}(index);

k = k+1;

end

filteredGSRData{2}(i) = median(subset);

end

% Display the unfiltered GSR data

figure;

subplot(2, 1, 1);

plot(GSRData{1}, GSRData{2});

title('GRAPH OF GSR DATA BEFORE MEDIAN FILTRATION');

xlabel('Time (s)');

ylabel('GSR Values (uS)');

% Display the filtered GSR data

subplot(2, 1, 2);

plot(filteredGSRData{1}, filteredGSRData{2}, 'r');

title('GRAPH OF GSR DATA AFTER MEDIAN FILTRATION');

xlabel('Time (s)');

ylabel('GSR Values (uS)');

%Saves current figure

saveas(gcf, 'Med_filt', 'jpg');

end

%The following function creates a n-point moving average filter for data set. It

%accepts two arguments: N-the moving average order and GSRData- the GSR data 3D cell array.

%and returns a filtered 3D cell array

function filteredGSRData = nPointMovingAverageFilter(N, GSRData)

filteredGSRData = GSRData;

%Calculate N-point moving avergae for other data elements

for i = 1:length(GSRData{2})

% Determine the subset based on the specified order

subset = zeros(1,N); % Reserves memory for window GSR values

k = 1; %Index variable for subset array

for j = -floor((N-1)/2):ceil((N-1)/2)

index = i + j;

index = max(1, min(index, length(GSRData{2}))); % Ensure index is within bounds

subset(k) = GSRData{2}(index);

k = k+1;

end

filteredGSRData{2}(i) = mean(subset);

end

% Display the unfiltered GSR data

figure;

subplot(2, 1, 1);

plot(GSRData{1}, GSRData{2});

title('GRAPH OF GSR DATA BEFORE N-POINT MOVING AVERAGE FILTRATION');

xlabel('Time (s)');

ylabel('GSR Values (uS)');

% Display the filtered GSR data

subplot(2, 1, 2);

plot(filteredGSRData{1}, filteredGSRData{2}, 'r');

title('GRAPH OF GSR DATA AFTER N-POINT MOVING AVERAGE FILTRATION');

xlabel('Time (s)');

ylabel('GSR Values (uS)');

%Saves current figure

saveas(gcf, 'Moving_Average', 'jpg');

end

%The following function creates a lowpass filter for data set. It

%accepts two arguments: Fp-the passband frequency and GSRData-the GSR data 3D cell array

%and returns a filtered 3D cell array.

function filteredGSRData = lowPassFilter(fp, GSRData)

% GSRData{1} contains datetime values and GSRData{2} contains GSR values

filteredGSRData = GSRData;

% Calculate the time difference between consecutive time points

timeDifference = diff(GSRData{1});

% Define the window size in seconds

meanWindowSize = 1/fp;  % Fp Hz for a 1/Fp s window

% Calculate the number of data points for the window

meanWindowPoints = round(meanWindowSize / median(timeDifference));

% Apply mean filter

filteredGSRData{2} = smoothdata(GSRData{2}, 'movmean', meanWindowPoints);

% Display the unfiltered GSR data

figure;

subplot(2, 1, 1);

plot(GSRData{1}, GSRData{2});

title('GRAPH OF GSR DATA BEFORE LOW PASS FILTRATION');

xlabel('Time (s)');

ylabel('GSR Values (uS)');

% Display the filtered GSR data

subplot(2, 1, 2);

plot(filteredGSRData{1}, filteredGSRData{2}, 'r');

title('GRAPH OF GSR DATA AFTER LOW PASS FILTRATION');

xlabel('Time (s)');

ylabel('GSR Values (uS)');

%Saves current figure

saveas(gcf, 'Lowpass', 'jpg');

end

%The following function normalizes GSR values to fit a 0-100 range. It

%accepts one arguments: GSRData-the GSR data 3D cell array

%and returns a filtered 3D cell array.

function normalisedData = normaliseData(GSRData)

%Copy GSRData to normalised data array

normalisedData = GSRData;

for i = 1:length(GSRData{2})

normalisedData{2}(i) = ((GSRData{2}(i) - min(GSRData{2})) / (max(GSRData{2})-min(GSRData{2})))* 100 ;

end

% Display the unfiltered GSR data

figure;

subplot(2, 1, 1);

plot(GSRData{1}, GSRData{2});

title('GRAPH OF GSR DATA BEFORE NORMALISATION');

xlabel('Time (s)');

ylabel('GSR Values (uS)');

% Display the filtered GSR data

subplot(2, 1, 2);

plot(normalisedData{1}, normalisedData{2}, 'r');

title('GRAPH OF GSR DATA AFTER NORMALISATION');

xlabel('Time (s)');

ylabel('GSR Values (uS)');

%Saves current figure

saveas(gcf, 'Normalised', 'jpg');

end

%The following function finds 10 features using GSR Data values from a file, passed as

%an argument and returns a vector containing the data.

function F = gsrFeatures(GSRData)

%Mean of GSR signal (F1)

F(1) = mean(GSRData{2});

%Variance of GSR signal (F2)

F(2) = var(GSRData{2});

%Find maximum and minimum values and their corresponding times;

max_pk = islocalmax(GSRData{2});

min_pk = islocalmin(GSRData{2});

%Maximum and minimum values

maxima = GSRData{2}(max_pk);

minima = GSRData{2}(min_pk);

%Corresponding Maximum and minimum time values

max_t = GSRData{1}(max_pk);

min_t = GSRData{1}(min_pk);

% Trough-peak seperation threshold, calculated using GSR_FEAR data trough-peak separation values at 90% Percentile

tr_pk_sep_prctl = 16.305;

%Intitialise peak energy sum variubale to zero

F(3) = 0;

F(5) = 0;

%Preallocates memory to store peak ampltitudes and rise tiems

SIZE = length(maxima);

risetime = zeros(1,SIZE);

amplitude = zeros(1,SIZE);

%Finds the peak ampltidues and the risetimes using  trough-peak separation thereshold

for i = 1:SIZE

%Find closet trough to peak at current position

idx = find(min_t < max_t(i), 1, 'last');

if ~isempty(idx)

%Checks if ampltidues are above threshold

if ((maxima(i) - minima(idx)) > tr_pk_sep_prctl)

%Stores peak amplitides within range

amplitude(i) = maxima(i);

risetime(i) = max_t(i) - min_t(idx);

%Calculates peak amplitude sum

F(3) = F(3) + risetime(i);

%Calculates peak energy sum

F(5) = F(5) + (0.5 * amplitude(i) * risetime(i));

end

end

end

%Calculates peak amplitude sum

F(4) = sum(amplitude);

%Finds highest peak amplitude sum

[F(6), pk_idx] = max(amplitude);

%Finds corresponding rise time for highest peak amplitude

F(7) = risetime(pk_idx);

%Finds number of peak amplitude

%Counts non-zero elements because array was preallocated with zeros of

%size greater than the number of 'important' peak amplitudes

F(8) = nnz(amplitude);

%Calculates mean power of signal

F(9) = mean(GSRData{2}.^2);

%Calculates Bandwidth of signals

dSquares = sum((diff(GSRData{2})).^2);

squares = sum(GSRData{2}.^2);

F(10) = sqrt(dSquares/ squares)/(2*pi);

end

%The following function finds fear index usings 10 features stored in a vector, passed as

%an argument and returns a scalar of the featr index.

function FearIndex = calculateFearIndex(F)

%Calculate Fear

FearIndex = F(1) + (2*F(2)) + F(3) + (0.5*F(4)) + F(5) + (2*F(6)) + F(7) + (5*F(8)) + (0.001*F(9)) + (0.5*F(10));

end

STEP 5: Tests and Verification

Test case 1: GSR_FEAR.csv

, which are in agreement with the expected test case.

Test case 2: GSR_Baseline.csv

, which are in agreement with the expected test case.

The User’s Guide

- To execute the program, compile and run the code found in the file named Fear_Analyser.m.

- The user may change function parameters( like file name) by modifying the values of the variables found in the main MATLAB file (Fear_Analyser.m).

- He user may also change the median filter or moving-point average order, or the passband frequency if they wish.

- User may have to re-run the program each time they wish to analyse a new file.